ALESIS HR-16/HR-16B MIDI SYSTEM EXCLUSIVE FORMAT

The following information is provided as a guide for programmers wishing to modify the data received via MIDI from the HR-16 for the purpose of interchanging patterns from separate block dumps, modification of drum setups, MIDI channel assignments, etc.  Great care must be taken to insure that all modified addresses are valid, since one incorrect value (the length of a pattern, for example) could result in all data being lost in the HR-16.  These errors may not show up immediately, since the incorrect values may not be accessed by the HR-16 until a particular pattern or song is selected.  Therefore, it is recommended that any data manipulation programs be thoroughly tested after loading into the HR-16 by selecting and recording on many patterns and songs before assuming that the data is valid.

For any of the sysex commands to be transmitted or received, the SYSEX ENABLE function (MIDI/UTIL page 15) must be turned on.  All 11 possible commands will be transmitted in the following format:

HEX		COMMENTS

F0H		SYSTEM EXCLUSIVE STATUS BYTE

00H

00H

0EH		ALESIS I.D. NUMBER

01H		HR-16 I.D. NUMBER

00H-0AH		SYSEX COMMAND

...		DATA

F7H		EOX

The following sysex commands are transmitted and received by the HR-16:

00H	COMPLETE MEMORY DUMP

A system exclusive MIDI data dump from the HR-16 is initiated by holding the TAPE button down, pressing (and releasing) the left arrow button once, and then pressing the RECORD button.  This command is followed by a block of data representing the contents of the HR-16's memory.  In order to optimize the data transfer, 8 MIDI bytes are used to transmit each block of 7 HR-16 data bytes.  If the 7 data bytes are looked at as one 56-bit word, the format for transmission is eight 7-bit words beginning with the most significant bit of the first byte, as follows:

SEVEN HR-16 BYTES:

0:	A7 A6 A5 A4 A3 A2 A1 A0

1:	B7 B6 B5 B4 B3 B2 B1 B0

2:	C7 C6 C5 C4 C3 C2 C1 C0

3:	D7 D6 D5 D4 D3 D2 D1 D0

4:	E7 E6 E5 E4 E3 E2 E1 E0

5:	F7 F6 F5 F4 F3 F2 F1 F0

6:	G7 G6 G5 G4 G3 G2 G1 G0

TRANSMITTED AS:

0:	 0 A7 A6 A5 A4 A3 A2 A1

1:	 0 A0 B7 B6 B5 B4 B3 B2

2:	 0 B1 B0 C7 C6 C5 C4 C3

3:	 0 C2 C1 C0 D7 D6 D5 D4

4:	 0 D3 D2 D1 D0 E7 E6 E5

5:	 0 E4 E3 E2 E1 E0 F7 F6

6:	 0 F5 F4 F3 F2 F1 F0 G7

7:	 0 G6 G5 G4 G3 G2 G1 G0

In order to use the data properly, it must be decoded properly into HR-16 byte format.  The following list gives the data locations within the "unpacked" (decoded) block of data, starting with the first byte of the block being 000.  NOTE:  All absolute addresses must have an offset of 8200H added to them (e.g., an absolute pointer to a pattern that starts at 35AH should have the pointer value 855AH).

000H-0C7H	DON'T CARE

0C8H	MIDI channel

0C9H	Receive MIDI drum triggers (0=off, 1=on)

0CAH	Transmit MIDI drum triggers (0=off, 1=on)

0CBH	MIDI note assignment of CLICK (0-127)

0CCH	MIDI note assignment of KICK (0-127)

0CDH	MIDI note assignment of SNARE (0-127)

0CEH	MIDI note assignment of CLS HAT (0-127)

0CFH	MIDI note assignment of MID HAT (0-127)

0D0H	MIDI note assignment of OPEN HAT (0-127)

0D1H	MIDI note assignment of CLAPS (0-127)

0D2H	MIDI note assignment of PERC 3 (0-127)

0D3H	MIDI note assignment of PERC 4 (0-127)

0D4H	MIDI note assignment of TOM 1 (0-127)

0D5H	MIDI note assignment of TOM 2 (0-127)

0D6H	MIDI note assignment of TOM 3 (0-127)

0D7H	MIDI note assignment of TOM 4 (0-127)

0D8H	MIDI note assignment of RIDE (0-127)

0D9H	MIDI note assignment of CRASH (0-127)

0DAH	MIDI note assignment of PERC 1 (0-127)

0DBH	MIDI note assignment of PERC 2 (0-127)

0DCH	MIDI echo (0=off, 1=on)

0DDH	MIDI program select (0=off, 1=on)

0DEH	Clock mode (0=MIDI & internal, 1=internal only, 2=tape)

0DFH	MIDI clock out (0=off, 1=on)

0E0H	Auto start (0=off, 1=on)

0E1H	Click value

0E2H	Click in play (0=off, 1=on)

0E3H	Manual voice/tune/mix (0=off, 1=on)

0E4H	Pad dynamics (0-0AH)

0E5H	Song loop (0=off, 1=on)

0E6H	Sysex enable (0=off, 1=on)

0E7H-0EBH	Software version: 5 ascii bytes, starting with a space (20H) if an HR-16, 	or a "B" (42H) if an HR-16B, followed by a 4 digit version number (e.g., 	"2.00", or 32H, 2EH, 30H, 30H)

0ECH-F3H	DON'T CARE

0F4H	0

0F5H-FDH	DON'T CARE

0FEH	27H

0FFH	0B5H

100H-187H	DON'T ALTER(this can be DON'T CARE if manual voice/tune/mix is off)

188H-191H	DON'T CARE

192H-1A1H	0

1A2H-1FFH	DON'T CARE

200H	MSB of absolute pointer to pattern 00

201H	LSB of absolute pointer to pattern 00

202H	MSB of absolute pointer to pattern 01

203H	LSB of absolute pointer to pattern 01

204H	MSB of absolute pointer to pattern 02

205H	LSB of absolute pointer to pattern 02

"	   "                 "              "        "

2C6H	MSB of absolute pointer to pattern 99

2C7H	LSB of absolute pointer to pattern 99

2C8H-2CCH	DON'T ALTER

2CDH	LSB of absolute pointer to first byte past SONG 99 data

2CEH	MSB of absolute pointer to first byte past SONG 99 data

2CFH-2D0H	DON'T ALTER

2D1H	LSB of FF00H minus data in 0CDH & 0CEH

2D2H	MSB of FF00H minus data in 0CDH & 0CEH

2D3H-2D4H	DON'T ALTER

2D5H	Shuffle amount (0-24, 0=50%)

2D6H-2D7H	DON'T ALTER

2D8H	Tempo (20-255)

2D9H	Quant clock count (must correspond to Quant value 0-9: 96, 64, 48, 32, 	24, 16, 12, 8, 6, 1)

2DAH	Quant value (0-9)

2DBH-2FFH	DON'T CARE

300H-301H	DON'T ALTER

302H	MSB of absolute pointer to song 00

303H	LSB of absolute pointer to song 00

304H	MSB of absolute pointer to song 01

305H	LSB of absolute pointer to song 01

306H	MSB of absolute pointer to song 02

307H	LSB of absolute pointer to song 02

"	   "                 "              "        "

3C8H	MSB of absolute pointer to song 99

3C9H	LSB of absolute pointer to song 99

3CAH-3CCH	DON'T ALTER

3CDH-3D2H	DON'T CARE

3D3H-3D6H	DON'T ALTER

3D7H-3FFH	DON'T CARE

400H-?	PATTERN 00 DATA

The pattern and song data must be dealt with in a specific manner:

1)	All pattern and song data must be in consecutive order, i.e., pattern 05 data cannot be before pattern 02's data.  The order for the data should be pattern 00 through 99, followed by song 00 through 99.

2)	If a pattern or song does not exist, its MSB pointer will = 0, which is an illegal pointer address.  Since there will be no data for this pattern, it is skipped, i.e., if pattern 04 is empty, pattern 05's data follows after pattern 03's data.

3)	There can be no gaps in the data.  Pattern 01's data must follow directly after pattern 00's data, etc.

4)	Locations 2CDH-2CEH (start of free memory) and 2D1H-2D2H (length of free memory) must be kept valid.

5)	Song 99 MUST exist.  This means that at least an empty song (03H 00H FFH) must be at the end of memory, and song 99 must point to it.

6)	It is suggested that all DON'T CARE data be left in the state that it is received in, although these locations can be replaced with zeroes.

PATTERN DATA FORMAT

The following is the format of each pattern, starting with the address pointed to byte the absolute pointer to the pattern (offset by 8200H):

00H	LSB of number of bytes in pattern, including header.

01H	MSB of number of bytes in pattern, including header.

02H	LSB of number of beats in pattern in BCD format	        (0 beats =

03H	MSB of number of beats in pattern in BCD format 	        empty pattern)

04H	KICK sound number (0-30H)

05H	KICK output assign (bit 7) and volume (bits 0-6: 0-63H)

06H	KICK panning (bits 5-7: 0=L, 6=R) & pitch (bits 0-4:0=-16, 1FH=+15)

07H	SNARE sound number (0-30H)

08H	SNARE output assign (bit 7) and volume (bits 0-6: 0-63H)

09H	SNARE panning (bits 5-7: 0=L, 6=R) & pitch (bits 0-4:0=-16, 1FH=+15)

0AH	CLS HAT sound number (0-30H)

0BH	CLS HAT output assign (bit 7) and volume (bits 0-6: 0-63H)

0CH	CLS HAT panning (bits 5-7: 0=L, 6=R) & pitch (bits 0-4:0=-16, 1FH=15)

0DH	MID HAT sound number (0-30H)

0EH	MID HAT output assign (bit 7) and volume (bits 0-6: 0-63H)

0FH	MID HAT panning (bits 5-7: 0=L, 6=R) & pitch (bits 0-4:0=-16, 1FH=15)

10H	OPN HAT sound number (0-30H)

11H	OPN HAT output assign (bit 7) and volume (bits 0-6: 0-63H)

12H	OPN HAT panning (bits 5-7: 0=L,6=R) & pitch (bits 0-4:0=-16, 1FH=15)

13H	CLAPS sound number (0-30H)

14H	CLAPS output assign (bit 7) and volume (bits 0-6: 0-63H)

15H	CLAPS panning (bits 5-7: 0=L, 6=R) & pitch (bits 0-4:0=-16, 1FH=+15)

16H	PERC 3 sound number (0-30H)

17H	PERC 3 output assign (bit 7) and volume (bits 0-6: 0-63H)

18H	PERC 3 panning (bits 5-7: 0=L, 6=R) & pitch (bits 0-4:0=-16, 1FH=+15)

19H	PERC 4 sound number (0-30H)

1AH	PERC 4 output assign (bit 7) and volume (bits 0-6: 0-63H)

1BH	PERC 4 panning (bits 5-7: 0=L, 6=R) & pitch (bits 0-4:0=-16, 1FH=+15)

1CH	TOM 1 sound number (0-30H)

1DH	TOM 1 output assign (bit 7) and volume (bits 0-6: 0-63H)

1EH	TOM 1 panning (bits 5-7: 0=L, 6=R) & pitch (bits 0-4:0=-16, 1FH=+15)

1FH	TOM 2 sound number (0-30H)

20H	TOM 2 output assign (bit 7) and volume (bits 0-6: 0-63H)

21H	TOM 2 panning (bits 5-7: 0=L, 6=R) & pitch (bits 0-4:0=-16, 1FH=+15)

22H	TOM 3 sound number (0-30H)

23H	TOM 3 output assign (bit 7) and volume (bits 0-6: 0-63H)

24H	TOM 3 panning (bits 5-7: 0=L, 6=R) & pitch (bits 0-4:0=-16, 1FH=+15)

25H	TOM 4 sound number (0-30H)

26H	TOM 4 output assign (bit 7) and volume (bits 0-6: 0-63H)

27H	TOM 4 panning (bits 5-7: 0=L, 6=R) & pitch (bits 0-4:0=-16, 1FH=+15)

28H	RIDE sound number (0-30H)

29H	RIDE output assign (bit 7) and volume (bits 0-6: 0-63H)

2AH	RIDE panning (bits 5-7: 0=L, 6=R) & pitch (bits 0-4:0=-16, 1FH=+15)

2BH	CRASH sound number (0-30H)

2CH	CRASH output assign (bit 7) and volume (bits 0-6: 0-63H)

2DH	CRASH panning (bits 5-7: 0=L, 6=R) & pitch (bits 0-4:0=-16, 1FH=+15)

2EH	PERC 1 sound number (0-30H)

2FH	PERC 1 output assign (bit 7) and volume (bits 0-6: 0-63H)

30H	PERC 1 panning (bits 5-7: 0=L, 6=R) & pitch (bits 0-4:0=-16, 1FH=+15)

31H	PERC 2 sound number (0-30H)

32H	PERC 2 output assign (bit 7) and volume (bits 0-6: 0-63H)

33H	PERC 2 panning (bits 5-7: 0=L, 6=R) & pitch (bits 0-4:0=-16, 1FH=+15)

34H-??	PATTERN DATA

??	0FFH (End of pattern)

Pattern data must follow these rules:

1)	Each byte of the pattern data (from 34H on) is either a drum event, or a count of clocks to wait, or 0FFH, which indicates the end of the pattern.  If the event is a drum event, bit 7 will be 0, bits 4-6 will contain the dynamics of the note (0-7), and bits 0-3 will contain the drum to be played (0-15, in the same order as above).  If the event is a count of clocks, bit 7 will be high, and bits 0-6 will indicate the number of clocks to wait (0-126).  A wait of 0 clocks is legal, and will cause the pointer to immediately advance to the next event.  An empty 8 beat pattern would consist of the following pattern data:  0FEH, 0FEH, 0FEH, 0FEH, 0FEH, 0FEH, 08CH, 0FFH.  This data will count clocks for 126*6+12 clocks (768 clocks, divided by 96 clocks per beat = 8 beats), afterwhich it will loop around since it has reached the end of the pattern.

2)	Adding the number of bytes in a pattern to the absolute pointer of a pattern should point to 1 byte past the last byte of the pattern.

SONG DATA FORMAT

The following is the format of each song, starting with the address pointed to by the absolute pointer to the pattern (offset by 8200H):

00H	LSB of number of bytes in song, including header.

01H	MSB of number of bytes in song, including header.

02H	Step 1

03H	Step 2

04H	Step 3

05H	Step 4

06H	etc....

xxH	Pattern number 0FFH (end of song)

Song data must follow these rules:

1)	Step data 0-99 indicate pattern steps.  Step data 100-250 indicate tempo change of 50% to 200%, respectively.  Step data of 251-254 are not accessible from the front panel, but would correspond to tempos of 201% through 204%.

2)	There cannot be more than 255 steps in a song.

3)	Adding the number of bytes in a song to the absolute pointer of a song should point to 1 byte past the last byte of the song.

01H	SELECT PATTERN OR SONG

This command is used to select a pattern or song.

0000000xB	x:  0=pattern, 1=song

0nnnnnnnB	n:  0-99 = pattern or song number

7FH		EOX

02H	RECORD BUTTON

This command is used to "press" or "release" the RECORD button.  It will affect whether the HR-16 will go into play or record when it receives a MIDI START command.  It does not affect what will happen when pressing play from the front panel.

0000000xB	x:  0=Record released, 1=Record pressed

F7H		EOX

03H	QUANTIZE AND SWING MODES

This command is used to set the quantize and swing modes.

0000xxxxB	xxxx:  0-9, quantize mode 0=1/4, 1=1/6, 2=1/8, 3=1/12, 4=1/16, 		5=1/24, 6=1/32, 7=1/48, 7=1/64, 9=1/384

000nnnnnB	nnnnn:  0-24 = swing amount, 0=50%

F7H		EOX

04H	SET PATTERN LENGTH

This command is used to change the length of the current pattern.

0t0ccccbB	t:  0=change end, 1=change top

0bbbaaaaB	ccccbbbbaaaa:  3 digit BCD length in beats (c=msb, a=lsb)

F7H		EOX

05H	COPY PATTERN, DRUM, OR SONG

This command is used to copy a pattern to another pattern, a drum to a pattern, or a song to a song.  If the command is to copy a pattern, the data received will be appended to the end of the currently selected pattern.  If the command is to copy a song, the data received will be appended to the end of the currently selected song.  If the command is to copy a drum, the drum number sent in the mode byte (-1) will be copied to the drum button hit within 3 seconds of receiving this command, otherwise it will be ignored.

000000xxB	x:  MSB of number of unpacked bytes of data after header
0xxxxxxyB	y:  LSB of number of unpacked bytes of data after header
0yyyyyyyB	if y_0, then x=x+1
000nnnnnB	n:  copy mode: 0=copy patt, 1-16=copy drum, 17=copy song
  |		The data is packed as 8 MIDI bytes per 7 data bytes as described
data		above in the MIDI bulk dump explanation.  It contains a complete
  |		pattern or song.

00000000B	This extra byte is required for proper operation
F7H		EOX

When transmitting any copy function, it is recommended that a pause of one second be inserted between the copy mode byte and the data that follows it to insure that the receiving HR-16 has enough time to move memory around to make room for the new pattern or song.  In addition, it is recommended that a delay be added before sending additional MIDI commands, since it may take a few seconds for the copy operation to be complete, during which time MIDI data will be ignored.  The amount of time required will be dependent on the length of the patterns or songs being copied, and the amount of free memory available.

06H	STEP EDIT MODE

This command is used to enter or exit step edit mode, as well as for stepping through the edited pattern.  If it is received while in song mode, it will be ignored.  If a step command is given while not in step mode, it will also be ignored.  The HR-16 will always transmit the current quantize and swing settings before sending an enter step command.  When receiving a step to next beat, any additional drums on the existing beat will be skipped.  When receiving a step to next drum command when currently on the last drum of a beat, the command will be ignored.

000000xxB	x:  0=enter step mode, 1=exit step mode, 2=step to next beat, 		     3=step to next drum

F7H		EOX

07H	REQUEST SYSEX DUMP

This command is used to request a complete sysex memory dump from the HR-16.  After receiving this command (if not in PLAY mode), the HR-16 will send out its complete memory via MIDI.

F7H		EOX

08H	CHANGE VOICE, TUNE, MIX SETTINGS

This command is receive only, and is used to change the voice settings of the current pattern.  It can function while the pattern is playing, and will update the display if the settings being displayed are changed.  It will be ignored if in song mode.  The format is packed as 8 MIDI bytes per 7 data bytes as described in the MIDI bulk dump explanation.  The data is identical to bytes 4 through 33H in the pattern data format.

data		48 data bytes, transmitted as 55 MIDI bytes

F7H		EOX

09H	MIDI COMMUNICATIONS TEST

This command is used to insure that bidirectional MIDI communication is functioning.  If an HR-16 receives the inquiry command (F0H, 00H, 00H, 0EH, 01H, 09H, 00H, F7H), it will respond with the response command (F0H, 00H, 00H, 0EH, 01H, 09H, 01H, F7H) only if MIDI ECHO is turned off.  If MIDI ECHO is on, the transmitting device will see the inquiry message returned to it if bidirectional communication is functioning.  This function is useful in determining that the HR-16 is ready to receive additional MIDI information.  For example, selecting a pattern, or copying a pattern can cause a delay in which the HR-16 will ignore incoming MIDI information if its input buffer overflows.  By sending this command after a copy pattern command (command 5), and then waiting for the response before sending any additional commands, it can be insured that the HR-16 will not miss any information.

0000000xB	x=0=MIDI inquiry, x=1=MIDI response

F7H		EOX

0AH	ERASE PATTERN OR SONG

This command is used to erase the currently selected pattern or song.  It will be ignored if in play mode.

F7H		EOX